Schematic

Modified DFE

There are some modification of DFE code. See the appendix for complete DFE Verilog code.

DC Level Shift

The original DFE is incorrect, and will cause DC level shift. The following two lines are changed.

// Wrong !! This will change DC level of both signal. // Use data_history = 00000 to verify this. // V(out) <+ slew(V(in) +tap1*(2*data_history[0]-1)+...; // V(outbar) <+ slew(V(inbar)-tap1*(2*data_history[0]-1)-...; // This is the correct one. V(out) <+ slew(V(in) +tap1*data_history[0]+...; V(outbar) <+ slew(V(inbar)-tap1*data_history[0]-...;

External DFE controller

I use external DFE controller to provide clock and reset. The clk and rst should be wire.

// Change from logical to wire, since I am using external DFE_controller, // which provides clk and rst signal. See layout. // logical clk, rst; wire clk, rst; ... always@(posedge(clk), rst) begin // Since rst is changed to wire, I have to explicitly specify rst == 1 condition. // if(rst) begin if(rst == 1) begin data_history <= 5'b00000; Dout <= 1'b0; End ...

Simulation

Lower corner are the PRBS or PWL voltage sources

Initial delay for both sources are 1ns.

Simulation setup

Maximum time step = 1ps. The larger time step will degrade eye, since DFE works on continuous time.

Maximum simulation time for eye measurement = 300 ns, or eye will be too coarse.

Pulse Response before DFE (Tapi = 0)

Differential Received Pulse Response and post cursors

| Time (ns) | 6.901 | 7.301 | 7.701 | 8.101 | 8.501 | 8.901 | Steady |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rx_Diff (mV) | 565 | 552 | 717 | 737 | 737 | 752 | 758 |
| Taps (m) |  | -103 | -20.5 | -10.5 | -10.5 | -3 |  |

| Rx_Diff (Zoomed) | Tx_Diff (Solid) , Rx_Diff (Dash) |
| --- | --- |

Pulse Response after DFE

To show the pulse response of DFE, the maximum time step of simulator must <=1ps, since DFE is working in continuous time. The resolution of DFE response is related to maximum time step significantly. Clk, Rst, and Dout signals also listed in figure.

| Time |  | (ns) | 6.901 | 7.301 | 7.701 | 8.101 | 8.501 | 8.901 | Steady |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rx_Diff | (Dash) | (mV) | 565 | 552 | 717 | 737 | 737 | 752 | 758 |
| Rx_Diff after DFE | (Solid) | (mV) | 566 | 756 | 756 | 756 | 756 | 756 | 757 |
| Dout |  |  | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

Eyes

Setup

Maximum time step = 1ps

Maximum simulation time = 300ps.

Eye Measure

|  | Tx Diff | Rx Diff before DFE | Rx Diff after DFE |
| --- | --- | --- | --- |
| Upper (mV) | 712 | 400 | 312 |
| Lower(mV) | -780 | -352 | -576 |
| Eye Height(mV) | 1492 | 752 | 888 |

We can see the improvement of DFE is roughly 130 mV, which is very close to tap1 of DFE, since the first ISI is much larger than the others.

Transmitted

Received Before DFE

Received After DFE

Time Domain Signal (Partial)

Dash: Before DFE , Solid: After DFE

Verilog Code

Modified DFE

//Verilog-AMS HDL for "ece546", "dfe_sampler2" "verilogams" `include "constants.vams" `include "disciplines.vams" module dfe_sampler2 (in, inbar, out, outbar, Dout, clk, rst); input in, inbar, clk, rst; output Dout, out, outbar; reg Dout; electrical in, inbar, out, outbar; // Change from logical to wire, since I am using external DFE_controller, // which provides clk and rst signal. See layout. // logical clk, rst; wire clk, rst; parameter real tap1 = - 0.103; parameter real tap2 = - 0.0205; parameter real tap3 = - 0.0105; parameter real tap4 = - 0.003; parameter real tap5 = - 0.003; reg[4:0] data_history; analog begin // Wrong !! This will change DC level of both signal. Use data_history = 00000 to verify this, and negative slew rate should be negative(produce warning). // V(out) <+ slew(V(in) +tap1*(2*data_history[0]-1)+tap2*(2*data_history[1]-1)+tap3*(2*data_history[2]-1)+tap4*(2*data_history[3]-1)+tap5*(2*data_history[4]-1),1e11, 1e11); // V(outbar) <+ slew(V(inbar)-tap1*(2*data_history[0]-1)-tap2*(2*data_history[1]-1)-tap3*(2*data_history[2]-1)-tap4*(2*data_history[3]-1)-tap5*(2*data_history[4]-1),1e11, 1e11); // This is the correct one. V(out) <+ slew(V(in) +tap1*data_history[0]+tap2*data_history[1]+tap3*data_history[2]+tap4*data_history[3]+tap5*data_history[4],1e11); V(outbar) <+ slew(V(inbar)-tap1*data_history[0]-tap2*data_history[1]-tap3*data_history[2]-tap4*data_history[3]-tap5*data_history[4],1e11); end always@(posedge(clk), rst) begin // Since rst is changed to wire, I have to explicitly specify rst == 1 condition. // if(rst) begin if(rst == 1) begin data_history <= 5'b00000; Dout <= 1'b0; end else begin if(V(out) - V(outbar) > 0.2) Dout <= 1'b1; else if (V(out) - V(outbar) < -0.2) Dout <= 1'b0; data_history[4:0] = {data_history[3:0],Dout}; end end endmodule

DFE Controller

//Verilog-AMS HDL for "ECE546", "dfe_control_unit" "verilogams" `include "constants.vams" `include "disciplines.vams" `timescale 10ps/1ps // Make sure the transient maximum time step must <= 1ps to see the // significance of DFE, or the time response will not be accurate, // which degrade the performance of DFE. module dfe_control_unit (clk_int, rst_int); output clk_int, rst_int; parameter real delayUnit = 690.131; // delay is approx. 6.9013 ns parameter cycle = 20; // clock period (20*10ps) reg rst_int; reg clk; // generate the internal clk. initial begin clk = 1; end // The reset signal has to a cycle earlier than the real signal // Another half cycle guarantee the correctness. initial begin rst_int = 0; #(delayUnit - cycle/2 - cycle) rst_int = 1; #(cycle) rst_int = 0; end always #(cycle/2) clk = ~clk; //Delay the clk to clk_int wire clk_int; assign #(delayUnit - $floor(delayUnit/cycle)*cycle) clk_int = clk; endmodule